National Translations of Euroclasses for Reaction to Fire

(EC decision 2000/147/EC)  This table has been re-written 2003-12-06, revision No. 6, 10th May 2004

Part 1: EU Member States										Please send corrections and additions by e-mail to

Part 2: EFTA Member States										bc-consulting@attglobal.net

| Part 1: EU Member State | Existing National Classification | EU-Classification | Progress in the Implementation | Notes See also #) footnotes |
| --- | --- | --- | --- | --- |

| Austria | Class A Class B1 Class B2 Class B3 | A1-A2-part of B B-C-D-part of E Part of C-D-E F | Önorm 3806 issued 2001-04-23, see #01) | #01) |
| --- | --- | --- | --- | --- |
| Belgium | A0 A1 A2 A3 A4 | A2-s1 B-s3 or C-s2 C-s3 D-s3 F | No decision is taken by the Belgian ministry see #02) | #02) |
| Cyprus |  |  | No info | No info |
| Czech Republic | A B C1 C2 C3 | A1 A2 B C or D E or F | EC decision adopted July 2002 |  |
| Denmark | Non-combustible Class A Class B ‘Combustible’, see #03) | A2-s1,d0 B-s1,d0 D-s2,d2 All other products | EC decision adopted 1st March 2002 | #03) |
| Estonia |  |  | No info | No info |
| Finland | Non Combustible Limited Combbustible Lining class I/I Lining class I/II and I- Lining class 2 Lining class -/- | A2-s1,d0 or A1 A2-s1,d0 see #04) B-s1,d0 C-s2,D1 d-s2,d2 F | New regulations in force 1st July 2002 | #04) |
| France | Non-Combustible M0 M1 M2 M3 M4 (non gouttant) M4 Not classified | A1 A1 or A2-s1,d0 A2-s1/2/3, d0/1 or B-s1/2/3, d0/1 C-s1/2/3,d0/1 D-s1/2/3,d0/1 D-s1/2/3,d0 All classes other than E-d2 & F E-d2 & F | Effective March 2003, Annes 4 to decision 31 December 2002 |  |
| Germany | Nichtbrennbar Schwerentflammbar/ No smoke, no droplets Schwerentflammbar/ Smoke, no droplets Schwerentflammbar/ Droplets, no smoke Schwerentflammbar/ Smoke & droplets Normalentflammbar/ Smoke, no sroplets Normalentflammbar/ Smoke & droplets Leichtentflammbar | A1 & A2-s1,d0 B-s1,d0 & C-s1,d0 A2-s2/3,d0 & B+C-s2/3,d0 A2-s1,d1/2 & B+C-s1,d1/2 A2-s3,d2 & B+C-s3,d2 D-s1/2/3,d0 and E D-s1/2/3,d2 and E-d2 F | Implemented. Official DIBt announcement in January 2002 |  |
| Greece |  |  | No info | No info |
| Hungary | A1 (non combustible) A2 (quasi non combustible) B1 (hard combustible) B2 (easy combustible) B3 (easy combustible) C0 (no melting) C1 (melting, no droplets) C2 (burning droplets/ignition) F0 (no smoke) F1 (moderate smoke) F2 (strongly smoke products) | #07) | At this time new regulations are under elaboration. Deadline May 2004. The classes are not expected to change before the end of 2004. See also #07) | #07) |
| Ireland |  |  | No official implementation is published at this time |  |
| Italy |  |  | Two different drafts are for discussion, no decision is taken |  |
| Latvia |  |  | No info | No info |
| Lithuania |  |  | No info | No info |
| Luxembourg |  |  | No info | No info |
| Malta |  |  | No info | No info |
| Netherlands | Non Combustible Class 1 Class 2 Class 3 Class 4 | A1 A2-s2 B-s2 C-s2 D-s2 | Publication on the Dutch official journal June 2003 NEN-EN 13501-1 |  |
| Poland | Nom combustible, and no droplets Non combustible Non ignitable, and no droplets Non ignitable Hard ignitable Easy ignitable Self extinguishing | A1 & A2s1/2/3,d0 A1 & A2 B-s1/2/3,d0 C and D-s1,d1/2/3 D-s2 and E and F E | Implementation late 2003 as annex to PN EN 13501-1 See #08) | #08) |
| Portugal |  |  | No info | No info |
| Slovakia |  |  | No info | No info |
| Slovenia see #05) | Non-Combustible Class 1 Class 2 Class 3 Class 4 | A1 & A2 B C D E | No info concerning implementation | #05) |
| Spain | M0 M1 M2 M3 M4 | To be used all classes according EC 2000/147. No general equivalence for all requirements established between old and new classes | Implementation late 2003. | No info |
| Sweden | Non-combustible Class I Class II Class III | A2-s1,d0 B-s1,d0 C-s2,d0 D-s2,d0 | Implementation October 2002 |  |
| United Kingdom (England and Wales) | Non-Combustible Limited Combustible Class 0 Class 1 Class 2 | A1 or considered as A1 without further testing A2-s3,d2 or better B-s3,d2 or better C-s3,d2 or better D-s3,d0 or better | Implementation with the amendment 2002 to the Building regulations |  |
| Unites Kingdom (Scotland) | Non Combustible Class 0 Class 1 Class 3 | A1 or A2 and A1 without further testing B C D | Implemented 2001 |  |

| Part 2: EFTA Member State | Existing National Classification | EU-Classification | Progress in the Implementation | Notes See also #) footnotes |
| --- | --- | --- | --- | --- |

| Iceland |  |  | No info | No info |
| --- | --- | --- | --- | --- |
| Liechtenstein |  |  | No info | No info |
| Norway | Non Combustible Limited combustible Combustible | A2-s1,d0 A2-s1,d0 All other | Publication 14th May 2003, see www.be.no/beweb |  |
| Switzerland |  |  | see #06) | #06) |

Notes:
#01:	Smoke and droplets classification according ÖN  3806 Anhang B (Informative)

#02:	It seems as Belgium will follow the ‘German approach’ and not make a translation.

#03:	The class ‘Combustible’ (or class C) does not exists in official documents. It is added to this table to bring the survey for Denmark in line with e.g. Norway and Germany where this ‘non applicable’ class is mentioned in the building regulation.

#04:	For escape routes only (To be confirmed)

#05:	The Slovenian standard for non-combustibility JUS UJI 040 is identical to ISO 1183 and

the Slovenian standard for flame spread JUS UJI 040 is identical to BS 482, part 7.

#06:	Input from Swiss officials has to be studied. No final solution available.

#07:	The Hungarian Standard series MSZ 595 dealing with fire issues has been implemented into the national legislation

No work is going on at this time concerning translation of the national classes.

#08	Details concerning smoke and droplets have not found the final form.

File: CEPMC / FWG document 03078 now 04xxx 10 May 2004 / BC